NCR  7895 Scanner/Scale 

Follow the process outlined below to connect and configure the NCR 7895 Scanner/Scale for use with NCR Counterpoint.

## Install NCR  7895 Scanner/Scale Drivers and 123Scan for NCR Utility

Follow these steps to install the NCR 7895 Scanner/Scale Zebra_Scanner_SDK on your NCR Counterpoint workstation or POS terminal:

- Download Zebra_Scanner_SDK file  from https://www5.ncr.com//support/support_drivers_patches.asp?Class=retail_RealScan#scannerfirmwaretable and run the exe file to complete the installation.
(This will install the Zebra CoreScanner Driver as well that’s needed)

- Install the 123Scan for NCR Utility tool by downloading the file 123Scan_for_NCR_(64bit)_v5.04.0006.zip from https://www5.ncr.com//support/text/Peripherals/7895Utility.htm and run the exe file to complete the installation and then Reboot the computer or POS terminal.

Note:

- It is recommended to install the latest version (v5.04.0006 or higher, 64-bit) of the 123Scan utility

- The latest 7895 Firmware Plug-in for the 123Scan tool is available at
https://www5.ncr.com//support/text/Peripherals/7895Utility.htm

# Configure the NCR 7895 using NCR 123Scan Utility
To configure the NCR 7895 scanner using 123Scan utility, follow these steps:

  - Connect 7895 Scanner/scale using RJ50 To Powered USB 12V Cable
Note: RJ50 will go into the POS port in the scanner/scale and USB end will go into the POS terminal.

  - Launch 123Scan for NCR Configuration Utility.

  - From Start page, click Create new configuration file. The application opens a new untitled tab and displays options in creating a configuration file.

  - When it asks for Select an option to start a configuration file, select My scanner is connected via USB cable. The application displays the list of detected scanners and their firmware plug-in options.

  - Click the plug-in option for your scanner to start the Configuration Wizard.

  - Set the configuration name.

  - On the left pane of the 123Scan Configuration Wizard, select Cable connection. The application displays the USB interface settings.

  - Keep USB as the cable and select “IBM Table Top USB” in USB Device Type.

  - When finish, click Done

  - Once the file is created and configured, load the file to the scanner by clicking Load to Scanner option in the top bar.
The application displays the Load configuration window.

  - From the scanner list, select the scanner to configure. The application starts loading the configuration to the selected scanner. Select close.

  - Save by clicking Save file to PC in the top bar.

  - Close the 123Scan utility.

  - Open C:/Program files/Zebra technologies/Barcode scanners/Common>Config.xml file to make the following edit:
under: 
<SIMULATED_HID_KB_SETTINGS> 
Change the following setting to 1 from 0:
 <Enable>1 </Enable>

  - Save the Config.xml file.

  - Turn off the scanner and disconnect the cable from the terminal.

  - Reboot POS terminal/workstation and reconnect the cable

  - Launch the 123Scan Utility.

  - Reload your configuration file created in above steps.

  - In Data view tab, click to select your scanner. This will open a screen to select an operation mode.

  - Under Select an operation mode, click to open “USB OPOS…” . This will open Scan log screen.


Verify scale and scanner works as desired:

- Scan the bar code of an item on the scanner/scale. This should add a row under the scan log as “OPOS” in Com. Mode and scan output showing the UPC bar code

- Keep the item on the scanner/scale and click “Get Scale Weight” in the top bar to weigh the item.  This should add a row under the scan log as “OPOS” in Com. Mode and scan output showing the weight

## Configure Scale Settings in Counterpoint

Follow these steps to configure you NCR 7895 Scanner/Scale in NCR Counterpoint:

- From the NCR Counterpoint main menu, select Setup > Point of Sale > Devices to display the Device Management window, and then expand the store to which the workstation you are configuring is assigned.

- Select the workstation you want to configure and click the Add a Device button.

- From the Add a Device menu, select Scales, and then select “Other OPOS Scale”.

- Click the  button next to the OPOS Name field, select the “ZEBRA_SCALE”.

- Click Device Information Test button and test should be successful

If the test is unsuccessful, double-check the connection between the scale and your workstation, verify the configuration settings, and test the scale again.

If the test is successful, Save & Close.

## Configure Scanner Settings

The NCR 7895 Scanner does not require any additional configuration in Counterpoint.