Printer Dip Switch Settings for Serial Communications

| Bixolon 350Plus | Epson TM88 Printers | Epson TM-H6000III Printer (COM1/2 only) |
| --- | --- | --- |
| Bank 1 12345678 ON *** OFF ***** Bank 2 12345678 ON OFF ******** Serial Information Baud Rate 115200 Data Bits 8 Bits Parity Check None Stop Bit 1 bit Handshaking DTR/DSR **Note: - The "Serial I/F" switch on the back of the printer (just to the left of the serial cable connection) should be set to ON** | Bank 1 12345678 ON * ** OFF ** *** Bank 2 12345678 ON OFF ******** Serial Information Baud Rate 38400 Data Bits 8 Bits Parity Check None Stop Bit 1 bit Handshaking XON/XOFF | Bank 1 12345678 ON * OFF ****** * Bank 2 12345678 ON OFF ******** Serial Information Baud Rate 19200 Data Bits 8 Bits Parity Check None Stop Bit 1 bit Handshaking DTR/DSR **Note: When DSW 1-3=ON (Handshaking=Xon/Xoff), the MICR does not read the check number and does not release the check after it goes through. DTR/DSR is the recommended setting for MICR processing. However, with DTR/DSR set, powered serial port communication does not function. The DB9 serial port communication to the Point-of-Sale is recommended.** |